UPS-MONITOR-V2

Design Brief  •  Revision 3.1  •  2026-04-11

William Collis  •  github.com/wkcollis1-eng/DIY-LiFePO4-UPS

⚠  CRITICAL: VBUS tied to LOAD_OUT at U3 VIN. NEVER connect USB while 12V present — 12V backfeed risk to host PC. Disconnect 12V before USB. Flash once via USB then use OTA exclusively.

# 1.  Purpose

Supplemental monitoring board for a 12V LiFePO4 UPS protecting Home Assistant Green (~3W) and Xfinity XB7 modem (~14W). Replaces the Shelly Plus Uni — this board is a strict superset at approximately half the power draw (~243mW vs ~500mW). Adds real-time current, power, and hardware-accumulated energy/charge via a Texas Instruments INA228 (20-bit I2C power monitor with silicon energy registers), reported to Home Assistant via ESPHome over WiFi.

No external programmer required. Flashes via USB-C using the ESP32-C3 native USB Serial/JTAG interface (CDC). All subsequent updates via ESPHome OTA — USB access requires 12V disconnection.

# 2.  Revision History

| Rev | Date | Change |
| --- | --- | --- |
| 2.0 | 2026-04-10 | Initial design — 45×35mm |
| 2.1 | 2026-04-10 | Board enlarged to 55×40mm for DFM. Buck, INA228, ESP32 zones separated. |
| 3.0 | 2026-04-11 | Expanded to 65×50mm. Complete re-route: F.Cu power only / B.Cu all signals. USB notch X=36–53 depth=2mm. SW1=BOOT(33,46) SW2=RESET(43,46). 0 DRC crossings. Submitted to PCBWay. |
| 3.1 | 2026-04-11 | Fixed three critical errors found during DFM review: (1) R5 10Ω VBUS series filter missing — added at (40,32); (2) VBUS routing error — old stub hit SCL pad (Y=28.5) instead of VBUS pad (Y=27.5), corrected; (3) C8 value error — 100nF corrected to 1µF for τ=10ms EN RC. Also added R3 10kΩ INA228 address pulldown previously missing from board. 33 components total. |

# 3.  System Topology

AC Grid → Mean Well HDR-60-12 (13.3V float)

→ Pololu Ideal Diode

→ Cyclenbatt 10Ah LiFePO4

→ Victron BP-65 LVD (11.8V trip)

→ [J1 BATT_IN] ── THIS BOARD ── [J2 LOAD_OUT]

↑

INA228 shunt

R_SHUNT 10mΩ

↓

┌────────────────┴────────────────┐

[F3:2A]                           [F4:5A]

│                                 │

HA Green (~3W)                XB7 Modem (~14W)

Board inserted post-BP-65. Power derived from LOAD_OUT via internal AP63203 buck — board's own ~243mW is included in all shunt measurements (zero blind spot).

# 4.  Board Specifications

| Parameter | Value |
| --- | --- |
| Board dimensions | 65 × 50mm (Rev 3.x — expanded from 55×40mm in Rev 2.x) |
| Layers | 2 (F.Cu, B.Cu) |
| Substrate | FR4, 1.6mm |
| Copper weight | 2oz (70µm) both layers |
| Surface finish | ENIG |
| Solder mask | Green, both sides |
| Silkscreen | White, top side |
| Min signal trace | 0.25mm (Kelvin: 0.25mm matched pair) |
| High-current trace | 3.0mm (J1 → R_SHUNT → J2, no vias) |
| Via drill / outer | 0.4mm / 0.8mm |
| USB-C notch | X=36–53mm, depth=2mm (covers J4 shield pads) |
| Antenna keepout | F.Cu X=60–65mm — copper free all layers |
| GND pour | B.Cu solid fill + F.Cu stitching vias |
| IPC class | IPC Class 2 |
| Flying probe test | Yes |
| Manufacturer | PCBWay (contact: Aran) |

# 5.  Component List (33 total)

| Ref | Value | Package | Notes |
| --- | --- | --- | --- |
| U1 | ESP32-C3-MINI-1-N4 | Module LGA-54 | Integrated PCB antenna. Antenna faces right edge X=60–65mm. |
| U2 | INA228AIDGST | MSOP-10 | I2C addr 0x40. reset_on_boot: false — preserves ENERGY/CHARGE registers. |
| U3 | AP63203WU-7 | SOT-23-6 | 13.3V→3.3V buck. η≈87%, Tj=32°C typical. No heatsink needed. |
| R_SHUNT | WSLT2512R0100FEA | WSK2512 (6332) | 10mΩ 1% TRUE Kelvin 4-terminal. NOT WSL 2-terminal. |
| R1, R2 | 4.7kΩ 1% | 0402 | I2C SDA/SCL pullups |
| R3 | 10kΩ 5% | 0402 | INA228 A0/A1 address pulldown → GND. Sets I2C addr 0x40. |
| R4 | 100Ω 5% | 0402 | LED current limit |
| R5 | 10Ω 5% | 0402 | INA228 VBUS series filter. LOAD_OUT→R5→U2 pin9. RC pole with C9 at 160kHz. |
| R6 | 210kΩ 1% | 0402 | Buck FB top. Vout=0.8×(1+210k/68k)=3.27V |
| R7 | 68kΩ 1% | 0402 | Buck FB bottom |
| R8 | 10kΩ 5% | 0402 | ESP32 EN pullup to +3V3 |
| R9 | 4.7kΩ 5% | 0402 | DS18B20 OneWire pullup (required) |
| R10, R11 | 5.1kΩ 5% | 0402 | USB-C CC1/CC2 — device identification |
| C1 | 100µF 25V | Elec 6.3×5.4mm | Buck input bulk. Low ESR preferred. |
| C2 | 10µF 25V X5R | 0805 | Buck input ceramic |
| C3 | 22µF 10V X5R | 0805 | Buck output |
| C4 | 100nF 10V X7R | 0402 | INA228 VCC bypass |
| C5, C6 | 100nF 10V X7R | 0402 | ESP32 VCC bypass (×2) |
| C7 | 100nF 10V X7R | 0402 | Buck output HF bypass |
| C8 | 1µF 10V X5R | 0402 | ESP32 EN RC cap. τ=10ms with R8. MUST be 1µF not 100nF. |
| C9 | 100nF 10V X7R | 0402 | INA228 VBUS filter (with R5). Net: VBUS_FILT. |
| L1 | 4.7µH 2.6A | IHLP-5050EZ | Buck inductor. Low DCR. |
| TVS1 | SMAJ15CA | SMA | 15V bidirectional TVS. Input transient clamp. |
| J1 | WAGO 2060-452 | SMD 2P | BATT_IN. Wire entry left. |
| J2 | WAGO 2060-452 | SMD 2P | LOAD_OUT. Wire entry left. |
| J3 | PinHeader 1×03 | 2.54mm TH | DS18B20: VCC / DATA / GND |
| J4 | GCT USB4135-GF-A | Edge mount | USB-C. Body overhangs board edge — intentional. ⚠ Backfeed risk. |
| SW1 | Omron B3S-1000 | SMD | BOOT — GPIO9. 10mm from SW2. |
| SW2 | Omron B3S-1000 | SMD | RESET — ESP32 EN. 10mm from SW1. |
| LED1 | Blue 0402 | 0402 | Status LED. Active-low via R4. |

# 6.  Net Definitions

| Net | Voltage | Description |
| --- | --- | --- |
| BATT_IN | 11.8–13.3V | J1 pin1 — battery side of shunt |
| LOAD_OUT | 11.8–13.3V | J2 pin1 — load side, post-shunt. Also U3 VIN and USB VBUS node. |
| VBUS_FILT | ≈LOAD_OUT | R5 output → U2 pin9 VBUS + C9. Filtered 12V for INA228 VBUS sense. |
| SHUNT_HI | ≈BATT_IN | Kelvin sense+ → INA228 IN+. Zero current path. |
| SHUNT_LO | ≈LOAD_OUT | Kelvin sense− → INA228 IN−. Zero current path. |
| +3V3 | 3.27V | Buck output. R6=210kΩ, R7=68kΩ. |
| SDA | 3.3V logic | I2C data — GPIO8 |
| SCL | 3.3V logic | I2C clock — GPIO1. NOT GPIO9 (strapping pin). |
| DS18B20 | 3.3V logic | OneWire — GPIO2. 4.7kΩ pullup R9. |
| LED_K | 3.3V logic | Status LED cathode, active-low — GPIO3 |
| USB_DM / USB_DP | USB | Native USB — GPIO18/19 → J4. Routes around U1 bottom on F.Cu then via to B.Cu. |
| USB_VBUS | 5V or 12V | Tied to LOAD_OUT at U3 VIN. See critical warning. |
| USB_CC1/CC2 | — | 5.1kΩ R10/R11 to GND — USB-C device identification |
| BOOT_N | 3.3V logic | GPIO9 → SW1 only. Never shared with any peripheral. |
| EN_RC | 3.3V logic | R8 10kΩ pullup + C8 1µF RC (τ=10ms) + SW2 to GND |
| SW_BUCK | Switching | U3 switch node. High di/dt — isolated from sense traces. |
| GND | 0V | Common. B.Cu solid pour + F.Cu stitching vias. |

# 7.  Routing Strategy

## F.Cu — Power Only

| Net | Width | Notes |
| --- | --- | --- |
| BATT_IN | 3.0mm | J1 → TVS stub → R_SHUNT C1 terminal |
| LOAD_OUT | 3.0mm | R_SHUNT C2 → J2. Narrow 0.5mm exit from Kelvin zone → J2 and U3 VIN branch |
| SW_BUCK | 0.5mm | U3 SW → L1. Short, isolated from sense traces. No sense traces within 3mm. |
| +3V3 bus | 0.8mm | Horizontal spine at Y=10mm from L1 output to ESP32 / INA228 / bypass caps |
| VBUS_FILT | 0.3mm | R5 pad2 → F.Cu up to Y=27.5 → U2 pin9. Spur to C9 at Y=28. |
| SHUNT_HI/LO | 0.25mm | Kelvin matched pair. No vias. ±2mm length match. Isolated zone. |
| USB_DM/DP | 0.3mm | F.Cu: route below U1 at Y=36/37mm. Via to B.Cu for J4 approach. |
| BOOT_N / EN_RC | 0.3mm | F.Cu stubs below U1 → via at Y=32.5 → B.Cu to SW1/SW2 |

## B.Cu — All Signals + GND Pour

All signal nets on B.Cu: SDA, SCL, DS18B20, LED_K, EN_RC, BOOT_N, USB_DM, USB_DP, USB_CC1, USB_CC2, FB_BUCK, USB_VBUS. Solid GND pour. 24+ F.Cu stitching vias. B.Cu GND pour behind U1 right edge acts as antenna reflector per Espressif reference design.

Total tracks + vias: 162.  DRC crossings: 0.

# 8.  Key Design Decisions

| Decision | Rationale |
| --- | --- |
| INA228 not INA226 | ESPHome native ina2xx_i2c platform. reset_on_boot: false preserves hardware ENERGY/CHARGE accumulation registers across restarts — critical for long-term energy tracking. |
| AP63203 buck not LDO | 13.3V→3.3V at 400mA ESP32 WiFi peak = 4W dissipation on any LDO → thermal shutdown. Buck dissipates ~150mW (η≈87%), Tj=32°C. No heatsink, no thermal vias needed. |
| VBUS tied to LOAD_OUT | Simplest way to power U3 from USB during programming without separate regulator or diode. Permanent consequence: never connect USB while 12V present. Board operates on 12V in production; USB for initial flash only, then OTA. |
| R5 10Ω VBUS filter | INA228 datasheet requires series resistance between high-voltage rail and VBUS pin for ESD and transient protection. R5 + C9 form RC filter at 160kHz. Missing in Rev 3.0 — added in Rev 3.1. |
| C8 = 1µF (not 100nF) | τ = R8 × C8 = 10kΩ × 1µF = 10ms EN RC delay. Ensures +3V3 rail stabilises before ESP32 EN asserts. 100nF gives only 1ms — insufficient. Corrected in Rev 3.1. |
| F.Cu power / B.Cu signals | Eliminates all F.Cu crossing violations. Rev 2.1 had 94 DRC crossings with mixed routing. Clean separation: F.Cu = power (BATT_IN, LOAD_OUT, +3V3, SW_BUCK). B.Cu = all 14 signal nets. |
| 65×50mm board | Rev 2.1 at 55×40mm had 94 DRC crossings due to routing density. 65×50mm (44% more area) resolved all crossings with clean routing. Rev 3.0/3.1 board. |
| Kelvin 4-terminal shunt | WSLT2512 vs WSL: separate current and sense terminals. Sense traces carry zero current — no IR drop error in INA228 measurement. |
| Retire Shelly Plus Uni | V2 board measures voltage, current, power, energy, temperature — strict superset of Shelly. Board draws ~243mW vs Shelly ~500mW. |

# 9.  Critical Errors Found and Fixed in Rev 3.1

| Error | Impact | Fix |
| --- | --- | --- |
| R5 missing from board | INA228 VBUS pin directly exposed to 12V rail. No filtering. Violates INA228 datasheet requirement. C9 useless without R5. | Added R5 10Ω at (40,32). LOAD_OUT → R5 → VBUS_FILT net → U2 pin9 + C9. |
| VBUS routing error — stub hit SCL pad | F.Cu trace T(37,28.5→36.2,28.5) landed on pad7=SCL (Y=28.5) not pad9=VBUS (Y=27.5). INA228 VBUS was unconnected; SCL had a spurious LOAD_OUT connection. | Removed erroneous stub. New route: LOAD_OUT branch → R5 → via → F.Cu at Y=27.5 to U2 pin9. |
| C8 = 100nF instead of 1µF | τ=1ms EN RC — insufficient for +3V3 stabilisation. Risk of erratic boot or download mode failure. | Corrected to 1µF X5R 0402. τ=10ms with R8 10kΩ. |
| R3 missing from board | INA228 A0/A1 address pins floating — I2C address undefined, could conflict. | Added R3 10kΩ at (30,34). Both pads to GND → address 0x40 confirmed. |
| C9 net incorrect (GND/GND) | C9 was bypassing GND to GND — completely non-functional. No VBUS filtering. | C9 pad1 = VBUS_FILT, pad2 = GND. Now correctly filters VBUS_FILT to GND. |

# 10.  Monitor Board Power Budget

| Component | Typical (mW) | Peak (mW) | Notes |
| --- | --- | --- | --- |
| U1 ESP32-C3 | ~150 | ~330 | WiFi transmit peak |
| U2 INA228 | ~1.5 | ~1.5 | Negligible |
| U3 AP63203 buck | ~85 | ~150 | Conversion loss at η=87% |
| Misc (LED, pullups) | ~6 | ~6 | Always-on |
| Total board (DC) | ~243 | ~490 | Included in shunt measurement — zero blind spot |
| Annual wall cost | ~$3.10/yr | — | At $0.29/kWh |

# 11.  ESPHome Configuration Summary

i2c:

sda: GPIO8

scl: GPIO1          # NOT GPIO9 (strapping pin)

sensor:

- platform: ina2xx_i2c

model: INA228

address: 0x40

shunt_resistance: 0.010   # 10mΩ WSLT2512R0100FEA

max_current: 5.0

reset_on_boot: false      # Preserves ENERGY/CHARGE registers

current:

name: UPS Current

power:

name: UPS Power

bus_voltage:

name: UPS Voltage

energy:

name: UPS Energy

charge:

name: UPS Charge

- platform: dallas_temp

address: 0x...

name: UPS Temperature

# 12.  Functional Test Specification

Equipment: DC bench supply 12V/3A, digital multimeter. No firmware required for steps 1–6.

| Step | Test | Pass Criterion |
| --- | --- | --- |
| 1 | Visual — no solder bridges on U2 (MSOP-10 0.5mm pitch), U3 (SOT-23-6), U1 (LGA-54). C1 polarity correct. R5 present between LOAD_OUT trace and U2 area. | Visual pass |
| 2 | Apply 12V to J1 pin1, GND to J1 pin2. J2 open. | No smoke, no trip |
| 3 | +3V3 at C3 positive terminal | 3.25–3.35V |
| 4 | +3V3 at U2 pin10 (INA228 VS) | 3.25–3.35V |
| 5 | +3V3 at U1 pad2 (ESP32 3V3) | 3.25–3.35V |
| 6 | Supply current at 12V, J2 open | < 150mA |
| 7* | VBUS_FILT: measure voltage at U2 pin9 (after R5) | 11.5–13.5V (≈LOAD_OUT minus R5 drop) |
| 8* | I2C scan on SDA/SCL | ACK at 0x40 |

* Steps 7–8 require access to U2 pins or ESPHome firmware.

# 13.  Gerber Export Process

Always export via Python pcbnew PLOT_CONTROLLER API — never kicad-cli. kicad-cli silently omits zone fills producing empty copper layers (B.Cu 787B vs correct 44KB).

# Correct export sequence:

board = pcbnew.LoadBoard(PCB)

pcbnew.ZONE_FILLER(board).Fill(board.Zones())  # CRITICAL — must precede plot

board.Save(PCB)

pctl = pcbnew.PLOT_CONTROLLER(board)

# ... plot each layer via pctl.PlotLayer()

| File | Extension | Layer |
| --- | --- | --- |
| UPS-MONITOR-V2-F_Cu | .gtl | Front copper — power traces + GND pour (120KB) |
| UPS-MONITOR-V2-B_Cu | .gbl | Back copper — all signals + GND pour (44KB) |
| UPS-MONITOR-V2-F_Mask | .gts | Front solder mask |
| UPS-MONITOR-V2-B_Mask | .gbs | Back solder mask |
| UPS-MONITOR-V2-F_Paste | .gtp | Front paste / stencil |
| UPS-MONITOR-V2-B_Paste | .gbp | Back paste |
| UPS-MONITOR-V2-F_Silkscreen | .gto | Front silkscreen — all designators |
| UPS-MONITOR-V2-Edge_Cuts | .gm1 | Board outline — 65×50mm + USB notch X=36–53 |
| UPS-MONITOR-V2-F_Fabrication | .gbr | Fabrication notes |
| UPS-MONITOR-V2-F_Courtyard | .gbr | Courtyard |
| UPS-MONITOR-V2 | .drl | Drill (Excellon) |
| UPS-MONITOR-V2-job | .gbrjob | Gerber job manifest |

ℹ  PCBWay DFM: silkscreen overlaps on R10/R11, C5/C6, R1/R2 — cosmetic only (paired 0402s at 2mm spacing). Approved as-is.

# 14.  Known Issues / Open Items

| Item | Notes |
| --- | --- |
| USB backfeed risk | VBUS tied to LOAD_OUT by design. Permanent operational constraint. Documented in BOM, CPL, and this brief. Disconnect 12V before connecting USB. |
| Rev 3.1 not yet submitted to PCBWay | Rev 3.0 Gerbers were submitted. Rev 3.1 corrects R5, C8, C9, R3, VBUS routing. New Gerbers + CPL + BOM must be sent to Aran before fabrication proceeds. |
| Shelly Plus Uni retirement | Pending board commissioning and ESPHome sensor validation in HA. |
| 12.2V software shutdown automation | Not yet live-validated in production UPS (separate from this board). |
| ESPHome OTA post-flash | After initial USB flash, all updates via OTA. USB access requires 12V disconnection. |
| R_SHUNT_HI / R_SHUNT_LO | Phantom footprints from early routing — completely removed in Rev 3.1. Not in CPL or BOM. |